Networking Multiple Choice Questions

1. An IPv4 address consists of _______ bits.
A) 4
B) 8
C) 32
D) 64

2. In IPv4, class _______ has the greatest number of addresses in each block.
A) A
B) B
C) C
D) D

3. Identify the class of the following IPv4 address: 4.5.6.7.
A) A
B) B
C) C
D) none of the above

4. Identify the class of the following IPv4 address: 229.1.2.3.
A) A
B) B
C) D
D) none of the above

5. Identify the class of the following IPv4 address: 191.1.2.3.
A) A
B) B
C) C
D) none of the above

6. What is the result of ANDing 255 and 15?
A) 255
B) 15
C) 0
D) none of the above

7. What is the result of ANDing 0 and 15?
A) 255
B) 15
C) 0
D) none of the above

8. What is the result of ANDing 254 and 15?
A) 254
B) 14
C) 0
D) none of the above

9. What is the result of ANDing 192 and 65?
A) 192
B) 65
C) 64
D) none of the above

10. Which one is not a contiguous mask?
A) 255.255.255.254
B) 255.255.224.0
C) 255.148.0.0
D) all are

11. The number of addresses in a class C block is _______.
A) 65,534
B) 16,777,216
C) 256
D) none of the above

12. The number of addresses in a class B block is _______.
A) 65,536
B) 16,777,216
C) 256
D) none of the above

13. The number of addresses in a class A block is _______.
A) 65,534
B) 16,777,216
C) 256
D) none of the above

14. The number of addresses assigned to an organization in classless addressing _______.
A) can be any number
B) must be a multiple of 256
C) must be a power of 2
D) none of the above

15. The first address assigned to an organization in classless addressing _______.
A) must be a power of 4
B) must be evenly divisible by the number of addresses
C) must belong to one of the A, B, or C classes
D) none of the above

16. Which address could be the beginning address of a block of 32 classless addresses?
A) 2.4.6.5
B) 2.4.6.16
C) 2.4.6.64
D) none of the above

17. Which address could be the beginning address of a block of 16 classless addresses?
A) 2.4.6.5
B) 2.4.6.15
C) 2.4.6.62
D) none of the above

18. Which address could be the beginning address of a block of 256 classless addresses?
A) 2.4.6.5
B) 2.4.6.15
C) 2.4.6.0
D) none of the above

19. What is the first address of a block of classless addresses if one of the addresses is 12.2.2.76/27?
A) 12.2.2.0
B) 12.2.2.32
C) 12.2.2.64
D) none of the above

20. What is the first address of a block of classless addresses if one of the addresses is 12.2.2.76/10?
A) 12.0.0.0
B) 12.2.0.0
C) 12.2.2.2
D) none of the above

21. What is the first address of a block of classless addresses if one of the addresses is 12.2.2.127/28?
A) 12.2.2.0
B) 12.2.2.96
C) 12.2.2.112
D) none of the above

22. Find the number of addresses in a block of classless addresses if one of the addresses is 12.2.2.7/24.
A) 32
B) 64
C) 256
D) none of the above

23. Find the number of addresses in a block of classless addresses if one of the addresses is 12.2.2.7/30.
A) 2
B) 4
C) 8
D) none of the above

24. What is the last address of a block of classless addresses if one of the addresses is 12.2.2.127/28?
A) 12.2.2.16
B) 12.2.2.112
C) 12.2.2.127
D) none of the above

25. What is the last address of a block of classless addresses if one of the addresses is 12.2.2.6/30?
A) 12.2.2.2
B) 12.2.2.6
C) 12.2.2.7
D) none of the above

26. An organization is granted a block; one address is 2.2.2.64/20. The organization needs 10 subnets. What is the subnet prefix length?
A) /20
B) /24
C) /25
D) none of the above

27. An organization is granted a block; one address is 2.2.2.64/25. If the subnet prefix length is /28, what is the maximum number of subnets?
A) 2
B) 4
C) 8
D) none of the above

28. An organization is granted a block of classless addresses with the starting address 199.34.76.64/28. How many addresses are granted?
A) 8
B) 16
C) 32
D) none of the above

29. An organization is granted a block of classless addresses with the starting address 199.34.76.128/29. How many addresses are granted?
A) 8
B) 16
C) 32
D) none of the above

30. An organization is granted a block of classless addresses with the starting address 199.34.32.0/27. How many addresses are granted?
A) 8
B) 16
C) 32
D) none of the above

31. What is the default mask for class A in CIDR notation?
A) /9
B) /8
C) /16
D) none of the above

32. What is the default mask for class B in CIDR notation?
A) /9
B) /8
C) /16
D) none of the above

33. What is the default mask for class C in CIDR notation?
A) /24
B) /8
C) /16
D) none of the above

34. In classless addressing, the ______ is another name for the common part of the address range.
A) suffix
B) prefix
C) netid
D) none of the above

35. In classless addressing, the _______ is the varying part (similar to the hostid).
A) suffix
B) prefix
C) hostid
D) none of the above

36. In classless addressing, the prefix length defines the _____________.
A) netid
B) hostid
C) mask
D) none of the above

37. In a block, the prefix length is /24; what is the mask?
A) 255.255.255.0
B) 255.255.242.0
C) 255.255.0.0
D) none of the above

38. In a block, the prefix length is /15; what is the mask?
A) 255.254.0.0
B) 255.255.255.0
C) 255.255.255.128
D) none of the above

39. In a block, the mask is 255.255.192.0; what is the prefix length?
A) /20
B) /28
C) /18
D) none of the above

40. An IPv6 address is _________ bits long.
A) 32
B) 64
C) 128
D) none of the above

41. An IPv6 address consists of ________ bytes (octets);
A) 4
B) 8
C) 16
D) none of the above

42. To make addresses more readable, IPv6 specifies ____________notation.
A) dotted decimal
B) hexadecimal colon
C) both a and b
D) none of the above

43. In hexadecimal colon notation, a 128-bit address is divided into _______ sections, each _____ hexadecimal digits in length.
A) 8: 2
B) 8: 3
C) 8: 4
D) none of the above

44. An IPv6 address can have up to __________ colons.
A) 8
B) 7
C) 4
D) none of the above

45. An IPv6 address can have up to __________ hexadecimal digits.
A) 16
B) 32
C) 8
D) none of the above

46. In IPv6, _______ address defines a single computer.
A) a unicast
B) a multicast
C) an anycast
D) none of the above

47. In IPv6, ____ address defines a group of computers with addresses that have the same prefix.
A) a unicast
B) a multicast
C) an anycast
D) none of the above

48. In IPv6, _________ address defines a group of computers.
A) a unicast
B) a multicast
C) an anycast
D) none of the above

49. In IPv6, the ________ prefix defines the purpose of the address.
A) type
B) purpose
C) both a and b
D) none of the above

50. In IPv6, the___________ address is generally used by a normal host as a unicast address.
A) provider-based unicast
B) link local
C) site local
D) none of the above

51. In IPv6, a _________address comprises 80 bits of zero, followed by 16 bits of one, followed by the 32-bit IPv4 address.
A) link local
B) site local
C) mapped
D) none of the above

52. In IPv6, a ___________ address is an address of 96 bits of zero followed by 32 bits of IPv4 address.
A) link local
B) site local
C) mapped
D) none of the above

53. In IPv6, a _______ address is used if a LAN uses the Internet protocols but is not connected to the Internet for security reasons.
A) link local
B) site local
C) mapped
D) none of the above

54. In IPv6, a ______address is used if a site with several networks uses the Internet protocols but is not connected to the Internet for security reasons.
A) link local
B) site local
C) mapped
D) none of the above